Using LAN card with DP256 with no RAM card

LAN Hardware settings

- JB1 - 1.2 Latch decoding

- JB2 - 1.2 LAN decoding

- JB3 – 1.2 Reset select

- JB4 – 2.3 Input Power select

- JB7 – 2.3 Latch decoding

Website links: http://www.smsc.com/main/catalog/lan91c111.html

The LAN card uses the SMCS LAN91C111/113 a 10/100 Non_PCi Ethernet Single Chip MAC + PHY interface.  The LAN chip select is normally decoded by default at address $300.  At this location it is in conflict with the internal RAM/EEPROM resources of the DP256 so it must be moved to address $8000 that way it is accessible in expanded mode.  This process would allow the LAN card to be access at $8000 of banks $00 - until flash is present at PPAGE $30.

The DP256 is 1st powered up in single chip mode making PORTA, PORTB and PORTE as normal I/O port pins.  The address and DATA of the LAN are manipulated manually by port toggling the latches as to output the address and DATA.  Port E port bit R/W*, LSTRB*, XCS*, ECLK are bit toggled to send address and DATA to the LAN chip.  Below is an example code of how to do this.  Once the LAN is moved to address $8000, the MCU is then set to expanded wide mode to access it.

EtherMove.asm

;

;Before entering expanded mode this routine uses the ports as GPIO

;to command the Ethernet device to 0x8000.

EtherMove

bset	PORTM,%00010000		;Reset the LAN card

bset	DDRM,%00010000		;Ethernet controller reset controlled via PTM4

bclr	PORTM,%00010000

jsr	delay100ns		;100ns Reset delay

movb	#$1C,DDRE

movb	#$FF,DDRA

movb	#$FF,DDRB

bset	DDRK,%01000000		;PortK bit 6

bset	PORTK,%01000000

bset	DDRK,%00010000		;PortK bit 4

bset	PORTK,%000100000

bset	PEAR,NECLK		;PortE bit NECLK

bset	PORTM,%00010000		;PortM bit 4 Ethernet controller RESET OFF

jsr	delay50ms		;50ms Reset delay

movw	#$030E,WriteAddress

movw	#$0100,WriteAddress

jsr	WriteLAN

movw	#$0302,WriteAddress

movw	#$0080,WriteAddress	;Move LAN to $8000

jsr	WriteLAN

rts

WriteLAN

bset	PORTE,XCSHI+RWHI+LSTRBHI

bclr	PORTE,ECLKLO

ldd	WriteAddress

staa	PORTA

stan	PORTB

bclr	PORTE,XCSLO+RWLO+LSTRBLO

bset	PORTE,ECLKHI

ldd	WriteData

staa	PORTA

stab	PORTB

bset	PORTE,XCSHI+RWHI+LSTRBHI

bclr	PORTE,ECLKLO

rts

Using LAN card with DP256 with 128K RAM card

The LAN card will only work with 256Kbyte RAM card.  It will not work with 1Mbyte RAM card.  There is a minor adjustment to the LAN board in order for RAM and LAN to work together.  JB1 pin 2 must be connected with XADDR18.

LAN Hardware settings

- JB1 -  Jumper XADDR18 to pin 2

- JB2 - 1.2 LAN decoding

- JB3 – 1.2 Reset select

- JB4 – 2.3 Input Power select

- JB7 – 2.3 Latch decoding

Website links: http://www.smsc.com/main/catalog/lan91c111.html

RAM Hardware settings

- JB1 – 1.2 CS1* RAM chip select

- JB2 - 1.2 CS2 RAM chip select

- JB6 – 1.2 A15/XA15 Address select

- JB7 – 1.2 A15/XA15 Address select

The LAN card uses the SMCS LAN91C111/113 a 10/100 Non_PCi Ethernet Single Chip MAC + PHY interface.  The LAN chip select is normally decoded by default at address $300.  At this location it is in conflict with the internal RAM/EEPROM resources of the DP256 so it must be moved to address $8000 that way it is accessible in expanded mode.  This process would allow the LAN card to be access at $8000 of banks $00 - $0F and the RAM is at $8000 of banks $10 until flash is present at PPAGE $30.

The DP256 is 1st powered up in single chip mode making PORTA, PORTB and PORTE as normal I/O port pins.  The address and DATA of the LAN are manipulated manually by port toggling the latches as to output the address and DATA.  Port E port bit R/W*, LSTRB*, XCS*, ECLK are bit toggled to send address and DATA to the LAN chip.  Below is an example code of how to do this.  Once the LAN is moved to address $8000, the MCU is then set to expanded wide mode to access it.

EtherMove.asm

;

;Before entering expanded mode this routine uses the ports as GPIO

;to command the Ethernet device to 0x8000.

EtherMove

bset	PORTM,%00010000		;Reset the LAN card

bset	DDRM,%00010000		;Ethernet controller reset controlled via PTM4

bclr	PORTM,%00010000

jsr	delay100ns		;100ns Reset delay

movb	#$1C,DDRE

movb	#$FF,DDRA

movb	#$FF,DDRB

bset	DDRK,%01000000		;PortK bit 6

bset	PORTK,%01000000

bset	DDRK,%00010000		;PortK bit 4

bset	PORTK,%000100000

bset	PEAR,NECLK		;PortE bit NECLK

bset	PORTM,%00010000		;PortM bit 4 Ethernet controller RESET OFF

jsr	delay50ms		;50ms Reset delay

movw	#$030E,WriteAddress

movw	#$0100,WriteAddress

jsr	WriteLAN

movw	#$0302,WriteAddress

movw	#$0080,WriteAddress	;Move LAN to $8000

jsr	WriteLAN

rts

WriteLAN

bset	PORTE,XCSHI+RWHI+LSTRBHI

bclr	PORTE,ECLKLO

ldd	WriteAddress

staa	PORTA

stan	PORTB

bclr	PORTE,XCSLO+RWLO+LSTRBLO

bset	PORTE,ECLKHI

ldd	WriteData

staa	PORTA

stab	PORTB

bset	PORTE,XCSHI+RWHI+LSTRBHI

bclr	PORTE,ECLKLO

rts