THE CIRCUT

My CPU board follows the circut described in the HC11

REFERENCE BOOK (available from Motorola).  Basically it's an HC11

wired in single chip mode.  I added a MAXIM MAX232 to enable

rs232 communications with a PC, and a MICROCHIP 25c010 16k serial

EEPROM for program storage.

I use a transistor, resistor network to act as a brownout

circut for RESET.  This is the circut on Motorola's EVB board.

However, if I had it to do again, I wold use a reset chip such as

Motorola's MC34064 or Dallas' econo reset DS1233.  What is

important here is that the circut has a way to reset when voltage

is too low and a push button reset switch.

To power the circut, I used 4 AA alkaline batteries,

unregulated.  Later I added an 78lc05 voltage regulator so I

could use 9v batteries and solar power.

There are other chips that will perform the voltage

translation that the MAX232 does.  Feel free to use them.  Also,

there are other sizes and makes of SPI format serial EEPROMs.

The software should work with those.  I read recently that there

are 4M byte versions of SPI format serial EEPROMs.  Imagine the

possibilities.

Finally, my origional version of SFORTH used a NATIONAL

93c86 2k SEEPROM, which is in MICROWIRE format.  MICROWIRE is

very similar to SPI, in fact I used the SPI port with a few small

software changes.  The code to drive the SEEPROM is not that much

bigger.  I switched to 25c04 because they were cheaper, and

easier to interface.  If you have a good source for NATIONAL

MICROWIRE type memories, you could make this system work with

them.  For that matter, I don't see why it wouldn't work with I2C

type SEEPROMs.  The code might be too big, but if you use enough

imagination, it might just work.

CONSTRUCTION

How you build the circut depends on what you buy.  The HC11

comes in a 52 pin plcc package and a 48 pin dip package.  The 48

pin package is easy to wire wrap.  The 52 pin package is more

difficult.  I found a good deal on wire wrap SIP sockets.  With

these I could fashion a wire wrap socket for the 52 pin chip.  It

is a tight fit and difficult to keep track of which pin is which,

but, it is easier than the point to point solder job I did on the

origional model.  Of course, you could buy a PC board for the

HC11 through various sources (see SFORTH.DOC).

A circut diagram is available in the file CIRCUT.BMP.  It

will work with PAINT in your WINDOWS APPLICATIONS program.