SFORTH INTERPRETER

INTRODUCTION

This is a brief description of SFORTH, a FORTH compiler for

the HC11 microcontroller.  SFORTH is designed to be compiled on

your personal computer, then downloaded to a serial EEPROM

connected to the HC11.  In the HC11's 512 bytes EEPROM, an

interpreter executes your code.

In writing SFORTH, I've tried to stick to the FORTH standard

as much as possible, so if you're familiar with FORTH, you'll

understand SFORTH.  In this document, I will describe a working

circut, the essential elements of the language, the compiler,

writing and downloading a program.

HARDWARE REQUIREMENTS

SFORTH runs on Motorola's 68hc11a1 (256 bytes RAM, 512 bytes

EEPROM) microcontroller.  The circut must be set up to run in

single chip, special bootstrap mode (MODA and MODB pins are

grounded).  A SPI compatible serial EEPROM should be connected to

the SPI port, pins PD2, PD3, PD4 and PD5.  For this example a

25c016 2k SEEPROM (available from DIGIKEY) was used , but other

sizes should work as long as they are SPI compatible.  Also

connected to the SCI port pins PD0 and PD1 is a MAX232 chip for

RS232 communications.  The software expects either a 4Mhz or 8Mhz

crystal.  It won't work otherwise.

There are a few boards available that can be easily modified

to fit this configuration.  LDG's board and the ADAPT-11 would

work.  Mondo Electronics also sells a PC board that would work

(you provide the parts). If you want to build your own, I've

included a circut diagram in this package that could be called up

from Windows.  See CIRCUT.BMP and CIRCUT.DOC.

The compiler, SFORTH.COM and loader, LOADER.COM should run

on any IBM compatible machine.  They're pretty small and won't

need more than 512k memory.  The HC11 communicates via the RS232

port.  The software only uses TX, RX and ground.

INSTALLATION

To run SFORTH on the HC11, the interpreter must be in it's

EEPROM.  To get it there, you need a program that will download

code to the EEPROM.  PCBUG is such a program.  PCBUG is a monitor

that requires no special hardware or a resident program in your

system.  It uses the special bootload program that Motorola

placed in every HC11 to download a small program into the HC11's

RAM. PCBUG will load the interpreter into EEPROM using the LOADS

command.  PCBUG is available free from the Motorola FREEWARE BBS.

Alternately your CPU might have the BUFALO monitor in it's

ROM.  I believe that versions above 3.0 will program the EEPROM.

You will need to be able to set your CPU into single chip mode,

MODA to ground, MODB to VCC.

To install the compiler and loader software on your PC, just

copy the files to the same directory.  The loader program needs

to have LOADER.S19 in the same directory in order to work

properly.

RUNNING THE COMPILER

SFORTH programs are written on any word processor.  Just

make sure the text is saved in DOS or ASCII format (that is no

control characters to confuse the compiler).  Also, make sure

that your program has the extension .FTH or the compiler won't

recognize it.  Once written, the program is compiled by typing:

SFORTH <NAME>

Leave out the extension.  If you forget to include the file,

SFORTH will prompt you for it.

If the compilation is successful, the compiler will print

"DONE." and a file <NAME>.S19 will be created with your code in

Motorola hex.  If there is an error, an error message will be

printed and no file will be created.

There are only three error messages.  "Undefined

word:<WORD>": you used a word that has not been defined

previously.  "Redefined word:<NAME>": you have redefined an

already defined word.  The compiler will ask if you want to

continue and quit if you don't. Finally, "FILE ERROR": the

compiler couldn't find the file you wanted to compile.

To use the loader, type:

LOADER <NAME>

Again leave out the extension.  As in SFORTH, you will be

prompted for the file if you leave it out of the command line.

You will be given the choice of DOWNLOAD, MONITOR or QUIT.

MONITOR listens in on the serial line from the HC11 and outputs

it in decimal form on screen.  If you want to change the baud

rate select BAUD and follow the prompts.

If you select DOWNLOAD, you will be asked for the crystal

frequency.  Next you will be told to reset the HC11.  The loader

downloads LOADER.S19 into the HC11's RAM then your compiled

program into the SEEPROM.  As it downloads the SFORTH program,

the code is echoed to the screen in decimal form.  When it's

done, Loader monitors the serial line and echoes it to the

screen.

PROGRAMMING IN SFORTH

While writing SFORTH, I tried to comply with standard FORTH

as much as possible.  There is a major difference.  In standard

FORTH, you can write your program and debug it as you go along.

FORTH is an interactive interpreter, you can run your program as

soon as you write it.  SFORTH, however is compiled, you have to

run it through the compiler before you can run it on the HC11.

Other than that, there are only a few differences between FORTH

and SFORTH.  If you can write a program in FORTH, you can write

one in SFORTH.

As in FORTH, there are two stacks, one for data and one for

return addresses.  They reside in the HC11's zero page RAM.

In SFORTH commands and subroutines are called WORDS.  You

write programs using the supplied words (see the GLOSSARY) or

making up your own.  To make your own word, start with ":" a

space and a name.  all words must be separated by white space,

that is spaces or returns or tabs.  SFORTH doesn't care if you

capitalize or not.

After you've named your word, you type in a series of

previously defined words to perform some task.  You end your

definition by typing ";" .

The general form is like this:

: <NAME> <words...> ; .

example:

: SEND begin SPSR c@ 80 and until SPDR c! ;

( this word sends a character out of the SCI port )

There are three types of words in SFORTH: compiler words,

interpreter words and macros.  Compiler words are instructions

for the compiler.  For instance, "BIN" tells the compiler that

all numbers after BIN are in binary form.  These words tell the

compiler to allocate space, give names to words and interact with

the PC.  Most compiler words must be executed outside of word

definitions.

Interpreter words are words that the interpreter recognizes.

For example "SWAP" exchanges the top two items on the stack.

These words tell the HC11 to do some task.  Interpreter words

must be within word definitions.

MACROS are combinations of interpreter words, some of which

are not usually available to the user.  They represent control

structures like IF/THEN or FOR/NEXT.  Like interpreter words,

then must be within word definitions.

PERIPHERALS

The HC11 has several peripheral devices that eliminate the

need to write special software.  There is an A/D converter, SCI

and SPI ports, and various timers.  All of these are accessible

as memory locations between $1000 and $103f.  The words "!", "@",

"C!", and "C@" are used to read and write to these locations.

EXAMPLE:

HEX ( all numbers in hexadecimal form )

102e CONSTANT SCSR ( define SCI Status Register )

102f CONSTANT SCDR ( define SCI Data Register )

: SEND ( start a WORD called SEND )

BEGIN ( start a BEGIN/UNTIL structure )

SCSR C@ ( fetch SCI port status )

80 AND ( mask off TDRE )

UNTIL ( if full do again )

SCDR C! ( transmit top of stack )

; ( end SEND )

: MAIN ( this is the last definition and will be executed upon

startup )

BEGIN

41 ( put "A" on stack )

SEND ( send it out )

0 ( put 0 on stack )

UNTIL ( will always loop back to BEGIN )

;

This example transmits an "A" through the SCI port.  First,

we define SCSR and SCDR as constants so that their value is put

on the stack when their name is encountered.  This makes the

program easier to read and debug.

The word "SEND" first polls the SCSR register to see if

there is a character in the transmit register (SCDR).  To do that

it reads the register and then masks off all the bits except bit

7 (80 in hex).  If it is set the transmit register is empty and a

new character can be sent out, otherwise it is still sending a

character and the result of "AND" will be 0.

"UNTIL" tests to see if the top of the stack is 0.  If it is

0, it branches back to "BEGIN" which of course repeats the

polling process.  If the top of the stack is not 0, that is if

bit 7 was 1, the code after "UNTIL" will be executed.

If SPDR is empty, the top of the stack is sent out of the

SCI port.  Since the SCDR register is one byte wide, use C!

(store one byte - the least significant).

"MAIN" puts the ASCII number for "A" (hex 41) on the stack

and then calls "SEND".  Then it puts 0 on the stack so that

"UNTIL" always branches back to "BEGIN".  If you didn't do this,

the HC11 will execute a "STOP" and halt the processor.

The compiler is designed to start execution at the last

defined word.  No special word is needed.  After the word is

executed the processor executes a SLEEP command stopping the

timers and halting execution until reset XIRQ or IRQ.

POLLING VS. INTERRUPTS

As the preceding example illustrates, there are control

registers associated with the peripheral registers.  These

registers give information about the status, or modes of various

ports, and enable certain features.  Also, they tell when a

process, say the main timer overflow, has occurred.

Often, we want to know when an event has occurred so that we

can do something else.  For instance, the Transmit Register is

empty, so now we can send another character.  There are two ways

of getting and using this information: polling and interrupts.

Polling continuously reads the register until a certain

condition is met and then continues on.  "SEND" in the previous

example does this.  The problem is that polling wastes the CPU's

time.  You might want to do other things while waiting for an

event to occur.  Also timing might be important.  Each

instruction in SFORTH takes between 280 and 900 microseconds to

perform (at 4 mhz).  In that time a critical event might have

occurred and been missed.

Using interrupts is a quicker way of responding to events.

When an event has occurred, immediately after the current HC11

instruction cycle, the HC11 stacks all of the registers and then

jumps to the routine that deals with that event.  When it's

finished, the registers are unstacked and execution continues

where it left off.

To use interrupts in SFORTH, first you have to write a

program to service the interrupt.  It is possible to write and

execute machine language programs with SFORTH.  See the section

on MACHINE LANGUAGE PROGRAMS.

Next you have to tell the HC11 where the program is.  When

the HC11 is in special bootstrap mode, the interrupt vector table

is located from $C4 to $FF in zero page RAM.  Each vector takes

up three bytes, a "jmp" opcode ($7E) and the address of the

routine.  You have to set up the vector or the HC11 won't know

where to go.

In order for the interrupt to happen, it has to be enabled.

There are two flags associated with interrupts.  Each maskable

interrupt (all interrupts except RESET, XIRQ and IRQ) has an

interrupt enable flag.  When it is set, the interrupt is enabled.

There is also 1a global interrupt flag in the Condition Code

Register.  This bit must be set in order for maskable interrupts

to occur.  Use "E_INT" to enable interrupts and "D_INT" to

disable them.

Writing interrupt service routines can be tricky.  Be sure

to get Motorola's HC11 REFERENCE book.  It's free from Motorola's

literature department.

In general, I've found that polling works fine if time is

not an issue.  For jobs requiring the timer features of the HC11,

you might consider using an interrupt.  For an example of using

interrupts, see TIMEOUT.FTH.

MACHINE LANGUAGE PROGRAMS

One of the nice features of the HC11 is that you can run

code from zero page memory.  This makes it possible to write

machine language programs in SFORTH.  Essentially, you will poke

the code into the RAM and then use "EXEC" to execute it.

First, to ensure that the code is protected, declare an

array big enough to hold your code.  Use the "DATA" word to

directly load the code into SEEPROM.  Then move the code from

SEEPROM to RAM (see the word "EMOVE" in TIMEOUT.FTH).  To run

your code, use the array's name and then "EXEC".

If you want to store data in variable space, it would be

helpful to know a bit about how the compiler reserves space for

variables.  When VARIABLE, CVARIABLE or ARRAY are invoked, the compiler

puts the name into the dictionary and reserves the required space in

ram.  Space is reserved in the order the space is declared, starting at

address 80 (50 hex).  You can calculate the address of the variable by

looking at the order that the space was declared.  For example:

CVARIABLE DAT1

6 ARRAY ARR1

VARIABLE DAT2

( the address of DAT1 is $50, ARR1 is $51, and DAT2 is $57 )

If you need to store or fetch data in your machine program, you

need to write the absolute address in your program, the compiler won't

accept a symbolic name for it.  Needless to say, a machine language

program requires some careful planning and attention to detail.  See

TIMEOUT.FTH and SERINT.FTH for examples of machine language and

interrupt routines.

VERY IMPORTANT: Your code must end in "jmp PUSHD" ($7E

$B775- push ACCD), "jmp PUSHX" ($7E $B776 - push ACCX) or "jmp

NEXT ($7E $B777 - the inner interpreter loop).  If it is an

interrupt service routine, it must end in "rti" ($3B).  Also,

before you return, you need to reset the interrupt flag for that

particular interrupt.  See the reference manual for details.

MEMORY MAP

RAM

00      IPH-INSTRUCTION POINTER HIGH BYTE

01      IPL-INSTRUCTION POINTER LOW BYTE

02      CI-CURRENT INSTRUCTION

03      EEDAT-EEPROM DATA-DATA FETCHED FROM OR TO EEPROM

04      RP-RETURN STACK POINTER

05      RETURN STACK-GROWS UP

-------------

4F      DATA STACK-GROWS DOWN

50      VARIABLE SPACE-GROWS UP - PUT MACHINE LANGUAGE ROUTINES HERE

---------------------

C4      INTERRUPT VECTORS - SEE HC11 REFERENCE MANUAL

FF      END OF RAM

B600-B7FF       FORTH INTERPRETER

GLOSSARY

COMPILER WORDS

The Compiler has one stack to store constants and compile

macros.  If you type a number or a constant it will be pushed

onto the compiler's stack.

:    - ( -- <NAME> ) Begins a compilation of a WORD.  When <NAME>

is called, the subroutine is performed.

Interpreter words will be compiled until ";"

is reached.

+   - ( a b -- a+b ) Adds the top two numbers on the compiler's

stack

-   - ( a b -- a-b ) Subtracts the top two numbers on the

compiler's stack.

*   - ( a b -- a*b ) Multiply the top two numbers on the

compiler's stack.

/   - ( a b -- q ) Divides the top two numbers on the compiler's

stack. Leaves the Quotient.

MOD - ( a b -- r ) Divides the top two numbers on the compiler's

stack.  Leaves the remainder.

(   - begins a comment.  Ignores text to ")".

ARRAY - ( n -- <NAME> ) reserves n bytes in RAM.  When <NAME> is

called, the address of the first byte of

the array is put on the stack.

BIN - All numbers following BIN are considered binary.

CONSTANT - ( n -- <NAME> ) When <NAME> is called, n is put on the

stack.

CVARIABLE - ( -- <NAME> ) Reserves one byte of RAM.  When

called, the  address of <NAME> is put

on the stack.

DATA - ( n -- <NAME> <a list of n numbers> ) Compiles n numbers

into SEEPROM.  When <NAME> is called, the address of the

first number is put on the stack.

DEC - All numbers following DEC are decimal.

HEX - all numbers following HEX are Hexadecimal.

LOAD - LOAD <NAME.EXT> Opens a program and compiles it to the

end of the file.  Use it like INCLUDE in "C" or BASIC.

VARIABLE - ( -- <NAME> ) Reserves space for a variable.  When

<NAME> is called, the address of <NAME>

is put on the stack.

INTERPRETER WORDS

The Interpreter has two stacks: the Return stack and the

Parameter stack (generally referred to as the "stack").  Numbers

and names (i.e. constants) are pushed on the parameter stack.

+ - ( a b -- a+b ) The sum of the top two numbers on the stack.

= - ( a b -- T|F ) If a=b then TRUE (-1) is put on stack, else

FALSE (0) is put on the stack.

> - ( a b -- T|F ) If a>b then TRUE else FALSE.

< - ( a b -- T|F ) If a<b then TRUE else FALSE.

1- - ( a -- a-1 ) Decreases the top of the stack by 1.

1+ - ( a -- a+1 ) Increases the top of the stack.

@ - ( addr -- data ) Loads data word at addr on stack.

! - ( data addr -- ) Stores data word in addr.

- - ( a b -- a-b ) Subtracts the top of the stack from the next

on the stack.

/ - ( a b -- q r ) Divides a by b and leaves the remainder and

quotient.

* - ( a b -- a*b ) Multiplies the lsb of a and the lsb of b.  Leaves an

8 bit result.

2* - ( a -- 2a ) Doubles the top of the stack.

+! - ( a addr -- ) Adds a to data in addr.

( - Begins a comment until ).

; - Ends the compilation of a WORD.

AND - ( a b -- a&b ) logical AND of top two numbers on stack

BIT_CLR - ( msk addr -- ) Clears bits indicated by msk in address

addr.

BIT_SET - ( msk addr -- ) Sets bits indicated by msk in addr.

C@ - ( addr -- byte ) Loads the byte at addr on the stack.

C! - ( b addr -- ) Stores a byte at addr.

D_INT - Disables interrupts.

DROP - ( a -- ) Drops the top of the stack.

DUP - ( a -- a ) Duplicates the top of the stack.

EXIT - Exit from a subroutine.

EE@ - ( addr -- data ) Loads the byte at addr in SEEPROM on the

stack.

EE! - ( data addr -- ) Stores data in addr in SEEPROM.

E_INT - Enables interrupts.

EXEC - ( addr -- ) Executes subroutine whose address is on top of

the stack.

HALT - ( -- ) Stops the timer and puts CPU into sleep mode until

RESET, XIRQ or IRQ.

NOT - ( a -- ^a ) One's complement of the top of the stack.

NEGATE - ( a -- -a ) Two's complement of the top of the stack.

OR - ( a b -- a|b ) Logical OR of a and b.

OVER - ( a b -- a b a ) copies second on the stack to the top of

the stack.

R@ - ( -- a ) Copies the top of the return stack and pushes it on

the stack.

>R - Pushes the top of the Parameter stack on the return stack.

R> - Pushes the top of the return stack on the parameter stack.

SWAP - ( a b -- b a ) The top of the stack switched with the next

on the stack.

SP@ - ( -- SP ) Pushes current stack pointer on stack.

XOR - ( a b -- a^b ) Logical XOR of a and b.

MACROS

These words are actually several interpreter words that

control program flow.

IF/ELSE/THEN - IF < words > THEN

or

IF < words > ELSE < words > THEN

If the top of the stack <> 0 then the words

between IF and THEN, or IF and ELSE are executed.

Otherwise the words between ELSE and THEN are

executed.

BEGIN/UNTIL - BEGIN < words > UNTIL.

The words after BEGIN are executed.  If the top of

the stack equals 0 then the program loops back to

BEGIN.

BEGIN/WHILE/REPEAT - BEGIN < condition > WHILE < words > REPEAT.

As long as < condition > is <> 0 then the words

between WHILE and REPEAT will be executed.

FOR/NEXT - n FOR < words > NEXT.

The words between FOR and NEXT will be repeated n

times.

LEAVE - Forces an early end to FOR/NEXT loop.

REFERENCES

MOTOROLA LITERATURE DISTRIBUTION

P.O.BOX 20912

PHOENIX, ARIZONA 85036

MOTOROLA FREEWARE BBS-1-512-891-3733

LDG ELECTRONICS

1445 PARREN RD.

ST. LENARD, MD.  20685

DIGIKEY

1-800-344-4539

call for a catalog

To order the ADAPT11:

TECHNOLOGICAL ARTS

1044 BAYVIEW AVE.

BOX 1704

TORONTO, ON. CANADA

M4G 3C2

MONDO-TRONICS

524 SAN ANSELMO AVE 107-13

SAN ANSELMO, CA. 94960